# Hardware Design Document

## Design Purpose

The goal of this product is to educate me, correct the frequency-response of my Logitech-X140 desktop speakers, and serve as a general-purpose audio EQ.

## Target Specifications and Motivations

- 3.5mm TS audio input and output

- ±15V DC input

- Rotary switches for frequency adjustments, potentiometers for gain and Q

- 2nd-order Butterworth HPF with adjustable cutoff (approx. 20Hz-120Hz)

- 4th-order Butterworth LPF with fixed cutoff (approx. 20kHz)

- LF shelving with adjustable corner (approx. 35Hz-220Hz) and adjustable gain (approx. ±8dB)

- HF shelving with adjustable corner (approx. 8kHz-10kHz) and adjustable gain (approx. ±8dB)

- Low-mid peaking filter with adjustable center frequency (approx. 100Hz – 1kHz), adjustable Q (approx. 0.5 – 5), and adjustable gain (approx. ±8dB)

- High-mid peaking filter with adjustable center frequency (approx. 1kHz – 10kHz), adjustable Q (approx. 0.5 – 5), and adjustable gain (approx. ±8dB)

- Audio pass switch

3.5mm TS audio communication is a standard analog output from computers and input into speakers. TS (mono) is chosen over TRS (stereo) for simplicity in design. This may be changed depending on budget.

The equalizer will be active to avoid large passive components, to have high input and output impedance, reduce cost, and because human ears are somewhat insensitive to phase shift. Assuming the voltage input has a maximum crest factor of 12 (link), tabulated below is the maximum amplification the eq can output without clipping based on RMS measurements at different volumes (see ./test) and the maximum output of the EQ. To design for a sensible ±8dB gain, it makes sense to power with ±15V power rails to give comfortable headroom for any audio type.

| Laptop Volume | RMS (mV) | Max EQ Voltage (V) | Max EQ gain without clipping (dB) |
| --- | --- | --- | --- |
| 0/100 | 0.76 | 9,12 | 60, 62 |
| 24/100 | 47 | 9,12 | 24, 27 |
| 50/100 | 173 | 9,12 | 13, 15 |
| 74/100 | 318 | 9,12 | 7, 10 |
| 100/100 | 536 | 9,12 | 3, 5 |

Rotary switches are chosen over potentiometers for tighter tolerance and repeatability for the user’s frequency adjustments, but potentiometers will give more specificity for sensitive factors like Q and gain.

The HPF will protect the EQ and speakers from signals <20Hz and cut out muddy bass up to 150Hz (any higher risks more significant phase shifts). The filter will be 2nd order for easier adjustment. The 4th LPF will greatly prevent high frequency signals that the speaker cannot reproduce but will not be adjustable because the attenuation and phase shift tends to leak into the music at low orders, which is needed for adjustability. Both the HPF and LPF should be Butterworth (Q=0.707) for minimal interference with the audio.

Shelving filters are chosen to broadly adjust bass and treble, and the gain is based on the above power specs.

Peaking filters are meant to cover the mid-range, which humans are the most sensitive to, so there is greater control. The Q factor ranges to allow wide, general changes and sharp attenuations (say to remove a resonant frequency) and the gain is based on the power specs.

The audio pass switch allows the audio to bypass the EQ filters for easy comparison.

These specs are somewhat inspired by fully parametric EQs like the GML 8200. Of course, this project is not meant to be comparable to a $9000 EQ.

## High-Pass Filter Design